Note: Leave the questions/part of the question in red.

Part – A

- Define depletion region.

- Define cutin voltage.

- Write the equation of Hall coefficient RH in terms of VH, w, B, and I.

- Reverse saturation current doubles for every ____________ rise in temperature.

- Define ripple factor.

- Write the expression for voltage regulation.

- Express α in terms of β.

- Define operating point.

- For fixed bias circuit, stability factor S = ________________.

- Define thermal runaway.

Part – B

- a)   Derive the expression for diffusion capacitance.

b)   A silicon diode has a reverse saturation current of 7.12 nA at room temperature.

Calculate its forward current if it is forward biased with a voltage of 0.7 V.          (3+2M)

(or)

- Derive the total current equation for a diode with help of diagrams.                 (5M)

- a)  Derive the following terms for a full-wave rectifier:

i) Ripple factor, ii) Efficiency.

b)  Explain the operation of a half wave rectifier with neat diagrams.                    (2+3M)

(or)

c)  Derive the expression for ripple factor of a full-wave rectifier with capacitor filter.                                                                                                                  (5M)

- a)  Explain the input and output characteristics of CE configuration with neat diagrams. (5M)

(or)

- Explain the voltage-divider bias configuration with neat diagram. Also find the stability factor S.									(5M)