Part-A

- Input impedance of MOSFET is negligible than BJT? True or False

- Draw small signal model of MOSFET.

- State any two features of power amplifiers.

- What is cross over distortion?

- Draw equivalent circuit of op-amp.

- List the non-ideal DC characteristics of an op-amp.

- Define CMRR of an op-amp.

- What is the slew rate of op-amp 741?

- Write the expression for gains of inverting and non-inverting amplifiers.

- Draw circuit diagram of zero crossing detector.

- State Barkhausen criterion.

Part-B

- What are the values of ID and gm for VGS = -0.8V if IDSS and VP are given as 12.4mA and -6V respectively?

- Draw and explain the MOSFET small signal model. Also derive transconductance from this model.

- Derive expressions for AV, Ri and Ro of common source amplifier.

- Describe the operation of common drain MOSFET amplifier and derive the equation for AV, Ri and Ro.

- Show that the transformer coupled class A amplifier maximum efficiency is 50%.

- Draw the circuit diagram of complementary symmetry class B amplifier and explain its operation. Also find its conversion efficiency. List its advantages over push-pull amplifier.

- Draw balanced differential amplifier in an op-amp and explain how it rejects common mode signals and amplifies difference mode signals.

- List and explain in detail ideal characteristics of an op-amp.

- Define slew rate. How fast can the output of an op-amp change by 10V, if its slew rate is 1V/μs?

- For an op-amp integrator circuit with R = 200MΩ and C = 0.1µF, sine wave input of 4Sin1000t is given. Determine the value of output voltage.

- Explain and draw the output waveforms of the ideal integrator circuit when the input is (i) sine wave  (ii) square wave    (iii) step input.

- Construct a Schmitt trigger using op-amp and explain its operation.

- Design a low pass filter using op-amp.

- Explain the operation of triangular waveform generator using op-amp.

- With help of neat circuit diagram and waveforms, explain the working of monostable multivibrator using op-amp. Also derive its pulse width.

- Design an op-amp based Wein Bridge oscillator for a frequency of 10kHz and explain its operation.