| S.No. | Question |
| --- | --- |
| 1a | Define race around condition. How would you eliminate the race around condition? |
| 1b | Illustrate the conversion of one flip-flop to another. Take conversion of JK flip-flop to D flip-flop and T flip-flop for illustration. |
| 2a | Design a 4-bit binary synchronous counter. |
| 2b | Build a 4-bit ring counter using D flip-flops with relevant timing diagrams. |
| 3a | Construct a sequence detector circuit, which detects three or more consecutive 1’s in a string of bits coming through an input line. |
| 3b | Reduce the number of states in the following state table and tabulate the reduced state table. PS; NS; Output. X=0; X=1; X=0; X=1. A; B; D; 0; 1. B; E; F; 0; 1. C; B; D; 0; 1. D; E; B; 0; 1. E; E; F; 0; 1. F; F; C; 0; 0. |
| 4a | Distinguish between Mealy model and Moore model. |
| S.No. | Question |
| 4b | Examine the set of maximal compatibilities with merger table for the given incompletely specified sequential machine. PS; NS, Z. I1; I2. A; E, 0; B, 0. B; F, 0; A, 0. C; E, _; C, 0. D; F, 1; D, 0. E; C, 1; C, 0. F; D, _; B, 0. |
| 4c | Name the elements of an ASM chart and draw each element. |